Indicators of rising damp

The tables below give examples of features of a house that can be affected by rising damp and describes how this may appear to an assessor. This will help an assessor to identify that the house is affected by damp and to distinguish what type of damp it is.

| Feature | Indicator | Description |
| --- | --- | --- |
| Walls (Exterior) | Discolouration | - Darkened colouring of bricks or stonework |
|  | Moss growth | - Moss growing near the bottom of the wall, but above the DPC |
|  | Deterioration | - Crumbling of the surface finish of stonework close to the ground |
| Walls (Interior) | Deterioration of wall fabric and finishings | - Discolouration of paintwork or wallpaper, normally in a regular pattern such as an almost horizontal tidemark - Soft, flaking or bubbling plaster on lower wall surfaces - Flaking and bubbling paintwork on lower wall surfaces - Loose wallpaper on lower wall surfaces - Skirtings affected by rot |
| House interior (all parts) | Odour | - Be aware of a dank moist smelling atmosphere in the room that could be coming from damp carpets, bed covers or mattress - Use this as an indication that there may be a damp problem in the room - Seek to find the source and cause of such a dampness problem |
|  | Fungi | - Look for fungi growth in all rooms, for example on walls, carpets etc. - This may indicate that there is a damp problem, or it may be the result of rot or other causes of dampness - Try and find the source of the problem to identify if it is due to rising damp |
| Wooden Floors | Rot | - Wood showing discoloration and may be crumbling or deteriorating - May be affected by rot - Boards failing to support weight when walked on - Dampness affecting carpet |

Causes of Rising Damp

The next table describes the main causes of a house being affected by rising damp and explains how an assessor can identify defects in the damp proof course ( DPC). Assessors should note that the absence of a DPC does not in itself mean the house is below tolerable standard. Visible indicators inside the house will normally be the trigger for a house failing the tolerable standard on the basis of rising damp

| Damp Proof Course ( DPC) | Absence | Some older houses were built without a DPC (and may not suit one) Basements converted into living accommodation may be below the DPC line Chimneys and hearth areas may not have a DPC and be more susceptible to rising damp problems |
| --- | --- | --- |
|  | Bridging | Look to see if the ground level is raised next to the wall Presence of spoil material against the base of the wall Retaining walls may be supporting earth and this can bridge the DPC |
|  | Damage | Deep structural cracks that may affect the damp proof course Deterioration of the waterproof barrier may allow water to rise unhindered into the building fabric Inappropriate repair to DPC and/or lower structures of houses with no DPC may allow rising damp to occur |

Indicators of Penetrating Damp The following table summarises the main areas of a house that may be affected by penetrating damp and describes how this may appear to an assessor. This will help an assessor to identify that the house is affected by damp and to distinguish what type of damp it is.

| Feature | Indicator | Description |
| --- | --- | --- |
| Walls (exterior) | Damp patches | - Look for discoloured or darkened damp patches on any part of the walls of the house - Any noticeable deterioration in the outward appearance of the brick of stonework |
|  | Moss growth | - Moss growth in a line down the wall, possibly below a plumbing overflow or defective guttering, indicates a persistent damp problem |
| Walls (interior) & ceilings | Damp patches | - Both walls and ceilings can be affected by penetrating damp - There may be an obvious damp patch because the walls or ceilings are discoloured - Wallpaper may be loose or bubbling, and paintwork can also deteriorate and flake off in the affected area - Plaster can deteriorate and lose its structure, feeling soft to the touch |
| Internal roof-space & wooden structures | Rot | - Examine the internal roof-space for evidence of water penetration - Wooden roof structures, including beams and supports, may be affected by rot - This will, in turn, affect the house's structural stability |
| House interior (all parts) | Odour | - Be aware of any bad or musty smell in the room that could be coming from damp carpets, bed covers or mattress - Use this as an indication that there may be a damp problem in the room - Seek to find the source and cause of such a problem |
|  | Fungi | - Look for fungi growth in all rooms, for example on walls, carpets etc. - This may indicate that there is a damp problem, or it may be the result of rot or other causes of dampness - Try and find the source of the problem to identify if it is due to penetrating damp |

Causes of Penetrating Damp

The tables below describe some of main causes of a house being affected by penetrating damp and explain what defects an assessor should look for that may indicate water penetration.

| Feature | Indicator | Description |
| --- | --- | --- |
| Walls (exterior) | Pointing | - Crumbling, loose or missing pointing and render may allow water to pass into the interior fabric of the building - Inappropriate repair using cement mortar may force water into the fabric of the building |
|  | Cracking | - Cracks in exterior walls may let or draw water enter the internal fabric |
| Rainwater management | Roof | - Look for damaged, displaced or missing tiles on the roof |
|  | Flashings | - Damaged flashings may allow water to seep underneath and into the internal fabric |
|  | Guttering & down-pipes | - Damaged, missing or displaced guttering or down-pipes may deflect excessive amounts of rainwater onto the exterior walls - Guttering that is blocked with leaves, moss, silt or vegetative growth will not be able to adequately manage the rainwater |